CLIENT IMPLEMENTATION REVIEW Working Packet Tracer Implementation & Verification Report ¦ Home Affairs Regional Office (Klerksdorp)

| STUDENT NAME SIBONISO MPEHLA | STUDENT ID 37277022 |
| --- | --- |
| CLIENT ID / PROJECT ID CLI-093 ¦ CMPG325-2026-093 | ASSIGNED NET BLOCK 10.38.0.0/16 |
| ASSIGNED FEATURE Port Security (Switchport Access Control) | CHANGE REQUEST CR5 (25% Host Growth Absorption) |

1. Working Packet Tracer File Overview

File Name: Home_Affairs_Klerksdorp_Implementation.pkt

Software Compatibility: Cisco Packet Tracer v8.2+

File Location in Repo: /topology/Home_Affairs_Klerksdorp_Implementation.pkt

Configured Network Devices:

• Router (R1-CORE): Cisco 2911 integrated with 802.1Q sub-interfaces for inter-VLAN routing.

• Access Switch (S1-ACCESS): Cisco Catalyst 2960 enforcing VLAN tags, trunking, and port security.

• Endpoints Configured: PC-IT-Admin (10.38.10.10/24), PC-Staff-01 (10.38.20.10/23), PC-Public-01 (10.38.30.10/24), Server-Main (10.38.40.10/27).

2. Assigned Feature Implemented: Port Security

VLAN 20 (Staff Operations - Fa0/3): Configured with maximum 2 dynamic sticky MAC addresses and Restrict violation penalty.

VLAN 30 (Public Counters - Fa0/4): Configured with maximum 1 dynamic sticky MAC address and Shutdown violation penalty.

! S1-ACCESS Port Security Implementation Script interface FastEthernet0/3 description Staff_Operations Workstation Port switchport mode access switchport access vlan 20 switchport port-security switchport port-security maximum 2 switchport port-security violation restrict switchport port-security mac-address sticky exit interface FastEthernet0/4 description Public_Counters Workstation Port switchport mode access switchport access vlan 30 switchport port-security switchport port-security maximum 1 switchport port-security violation shutdown switchport port-security mac-address sticky exit

3. Packet Tracer Testing & Verification Evidence

A. ICMP Ping Tests (Inter-VLAN &amp; Gateway Routing):

Execution: Executed directly from PC-Staff-01 Command Prompt to verify routing across subnets.

- [SCREENSHOT 1: Gateway Ping (10.38.20.1)]

- [SCREENSHOT 2: Server Ping (10.38.40.10)]

- [SCREENSHOT 3: IT-Admin Ping (10.38.10.10)]

Port Security Status Verification: Executed `show port-security interface Fa0/3` and `show port-security interface Fa0/4` confirming active enforcement.

Violation Penalty Simulation: Attached rogue device to Fa0/4. System immediately triggered `err-disabled` state on interface, shutting down the port.

4. Updated GitHub Portfolio Link

Repository URL: https://github.com/Siboniso-Mpehla/CMPG325-Home_Affairs_Klerksdorp

Repository Contents: Includes Home Affairs Klerksdorp.pkt, Milestone1_Client_Design_Review.docx,  Milestone2_Working_Packet_Tracer_Implementation_Review.docx.